[||TITLE||]

INTERFERENCE BETWEEN LIFTGATE AND REAR BUMPER

[||TSB_NUMBER||]

R067/20

[||DESCRIPTION||]

Some vehicles may exhibit that the liftgate may interfere with the rear bumper when closed forcefully resulting in damaged paint on the rear bumper.

CAUSE

Due to variations of the clearance between the liftgate and the rear bumper, the bottom edge of the liftgate may overrun to the rear bumper.

MASS PRODUCTION CHANGE

The hardness of stop rubbers on the liftgate has been modified.

SUBJECT VIN & PRODUCTION DATE RANGE

Japan Built

| Spec. | VIN Range |  | Production Date Range |
| --- | --- | --- | --- |
| Australia | JM0 BP **** ** | 100001 - 122710 | Jan. 15, 2019 – Jun. 25, 2020 |
| General (R.H.D.) | JM6 BP **** ** | 100001 - 105569 | Mar 19, 2019 – Jun. 10, 2020 |
| General (L.H.D.) | JM7 BP **** ** | 100001 - 112706 | Apr. 23, 2019 – May 28, 2020 |
| Israel | JMZ BP **** ** | 100001 - 109241 | May 26, 2019 – Nov. 28, 2019 |
| Europe (L.H.D. & UK Specs.) | JMZ BP **** ** JM4 BP **** ** | 100001 - 159715 | Nov. 6, 2018 – Jun. 25, 2020 |
| US (2019-2021MY) | JM1 BP **** ** | 100001 - 300045 | Nov.6, 2018 - Jun. 16, 2020 |

Thailand Built

| Thailand | MM8 BP **** ** | 300001 - 999999 | May 28, 2019 - TBD |
| --- | --- | --- | --- |

Vietnam Built

| Vietnam | RN2 B1**** ** RN2 B2**** ** RN2 B3**** ** RN2 B4**** ** RN2 B5**** ** RN2 B6**** ** | TBD |  |
| --- | --- | --- | --- |

[||REPAIR_PROCEDURE||]

Replace the stop rubbers with modified ones.

1

2

(1)	Rubber, Stop (Main)

(2)	Rubber, Stop (Sub)

NOTE: Explain to the customer below when replacing the stop rubbers.

By changing to the harder rubber, the overrun allowance between the liftgate and the weatherstrip is reduced and the interference is prevented.

As a result, the bounce when opening the liftgate becomes stronger, therefore the liftgate should be closed a little harder against the stop rubber.

The bounce and tightness of closing the liftgate will be reduced gradually as time goes by.

[||CALIBRATION||]

R.H.D.

| ECU | File Name |
| --- | --- |
| DOOR_ESU_D | B0J8-66S09-E |
| DOOR_ESU_P | B0J8-66S08-E |

L.H.D. except China

| ECU | File Name |
| --- | --- |
| DOOR_ESU_D | BCKA-66S09-E |
| DOOR_ESU_P | BCKA-66S08-E |

China

| ECU | File Name |
| --- | --- |
| DOOR_ESU_D | BELG-66S09-C |
| DOOR_ESU_P | BELG-66S08-C |

[||PARTS_INFORMATION||]

| Parts Number | Description | Qty. | Notes |
| --- | --- | --- | --- |
| TK78-62-877A | Rubber, Stop | 2 | Main |
| KB7W-62-877C | Rubber, Stop | 2 | Sub |

[[||WARRANTY_INFORMATION||]

| Symptom Code | 90 |
| --- | --- |
| Damage Code | 9H |
| Causal Part No. | TK78-62-877A |
| Quantity | 2 |
| Operation No. and Labor Hours | XXS46XRX / 0.2 Hrs. |
| Period Covered | Normal Warranty Period |

Enter KB7W-62-877C (Sub) as a related part number. If painting is required, claim it using sublet code “A1” or “P” (for New Zealand) attaching pictures of the damage.